Dexter is an elegant small deck light in aluminum designed for recessed installation in wooden decks or stone paving. It is also visually striking and functional for use as markers on stair steps or at the termination of a level on a terrace. Along a path, you can create a soft flow of light while illuminating the way.

The package includes 10 deck lights, a 10m splitter cable, and a transformer (21w). Feel free to add more lights and splitter cables if needed.

Don't forget the transformer. Choose a transformer that suits the total wattage of all your lamps. For optimal life we recommend that you load your transformer with no more than 85% of the max capacity.

Keep it simple! Add a photo cell that automatically turn the lights on in the evening and off again at dawn.

Garden Plug & Play by LightsOn 
A flexible 12-volt lighting system for your garden. Easy to install and easy to extend (max 50m) to reach all parts of your garden. Designed in Sweden for the tough Nordic climate, all components are made of durable and weather proof materials, guaranteeing a long maintenance free life.

Specification Dexter (one unit):
• 12 volt AC
• 0,5 watt LED (integrated light source)
• 20 lumen
• IP67
• Beam angle (110°) 
• Warm white light (3000°K)
• Aluminium, frosted acrylic glass
• Ø 32 mm diameter (26 mm drill)
• 70 cm cable
• Dimmable with Dimmer (art 5094) and SmartControl (art 5129)

Specification 21 watt transformer:
• 230 volt AC -> 12 volt AC
• 21-watt (21 VA), 18-watt maximum load
• IP44

Specification 10 m splitter cable:
• 10 meter long with 10 splitters (2 x 0,75 mm2) 
• Rubber cable 
• Quick connector 22 millimeter (largest diameter measurement) in each end