Orion 10-pack decklight with transformer

This is an all in one package containing 10 Orion deck lights, 10m splitter cable and a transformer.

LightsOn Garden Plug & Play. A flexible 12-volt lighting system for your garden. Easy to install and easy to extend (max 50m) to reach all parts of your garden. Designed in Sweden for the tough Nordic climate, all components are made of durable and weatherproof materials, guaranteeing a long maintenance free life.

Don't forget the transformer. Choose a transformer that suits the total wattage of all your lamps. For optimal life we commend that you load your transformer with no more than 85% of the max capacity.

Keep it simple! Add a photocell that automatically turn the lights on in the evening and off again at dawn.

Do you prefer a bit softer light? Orion works well together with the product Dimmer (art.5094)

Specifications Orion: 
• 12 volt AC 
• 5 x 0,5 watt LED (integrated light source) 
• 5 x 20 lumen 
• IP65 
• Narrow beam angle (15°) 
• Warm white light (3000°K) 
• Dimmable 
• Aluminium, frosted acrylic glass 
• Ø 35 mm diameter (26 mm hole) 
• 70 cm cable 
• Dimmable with LightsOn dimmer

Specifications 21 watt transformer: 
• 230 volt AC -> 12 volt AC 
• 21 watt (21 VA), maximum load 18 watt 
• IP44

Specifications 10 m splitter cable: 
• 10 meters long with 10 splitters (2 x 0,75 mm2) 
• Rubber cable 
• Quick connectors 22 millimeter diameter in each end